Command Line Options:

-h      Output in Intel Hex format

-l      Show listing

-s      Show symbols

-t      Suppres trailing information

-Sl     Sort label reference by line

-Sa     Sort label reference alphabetically

-u      Force output to uppercase

-V      Show Version

-v      Verbose output

-x      Show cross reference

-d name Specify assembly definition file

-r	Show rule that provided assembly match

this is used for debugging def files

-Dsym=val Define a preprocessor symbol

Definition files:

-----------------

The format of a definition file is as follows:

comment

options

class definitions

*

Assembly translations

*

The comment is the very first line of the file.  This line can contain anything

but usually describes what the definition file is for

Options are preceeded by the word OPTION.  Options provide a means to alter

the operation of the assembler.  The currently defined options are:

OPTION 16BIT  - This optin one increments the address pointer once for every

two bytes output

OPTION BIG    - Use big-endianess when writing out words (Default)

OPTION LITTLE - Ise little-endianess when writing out words

Class definitions allow you to specify multiple entities for an instruction

A common use is for the processor's register set.  A class entry is defined

as follows:

CLASS name

text=value[,text=value ...]

Each text entry is replaced by the value entry when the machine language

instruction is being built.

Assembly translations are each two lines.  The first line specifies the

pattern to be matched for the particular instruction.  The second line

specifies how the machine language instruction is built for the matched

pattern.  For example:

ADD	M

c1

This specifies that when an instruction 'ADD  M' is found in the source

file, a C1 byte is written out to the object file.

The instruction pattern is defined as follows.  The assembly mnemonic

begins the line, a tab is used before the arguments.  All characters

must match one for one, or when a character is preceeded by a \, the

wildcard function must match.  The following wildcard functions are

currently defined

\N         - Match a nybble (0-15)

\B         - Match a byte (0-255)

\W         - Match a word (0-65535)

\(n)       - Match a value of up to n bits. n is a single hex digit

\D         - Displacement.  If this byte is a label or a number not

starting with + or -, the value is taken as an address that

the current instruction pointer should be subtracted from

In this mode, the value of the instruction pointer is 1

following the actual displacement byte.  If the number

begins with + or - it is taken as the relative displacement

value.

\L         - Remaining text is placed into the list buffer as bytes

\M         - Remaining text is placed into the list buffer as words

\{class}   - Match an entry within the specified class

Each wildcard match places the matched value into a replacement variable

The first wildcard places the match in 1, the second in 2, and so forth.

These replacement variables can then be used in the construction line.

The construction line is used to build the machine language code for the

matched pattern.  The construction function begins with an accumulator of 0.

All characters on the construction line effect the accumulator.  When a blank

is reached the accumulator is output as a byte and then zeroed.  Multiple

byte instructions are built by separating each byte assembly with spaces.

An end of line will terminate the construction for the matched instruction.

The following characters affect the accumulator as follows:

0-f  - Hex digits.  Hex digits shift the accumulator by 1 nybble and then

the new digit is ORed with the accumulator. Example 1. C5 would

produce an output of C5.  C5A would output 5A, this occurs because

the accumulator is a single byte.

\n   - Use the value in the specified replacement variable.  Example: \1

would use the value from the first wildcard match

lo(n) - Get the low byte value of the specified replacement variable.

Example: lo(2) would get the low order byte from the second

wildcard replacement.  this will overlay the accumulator

hi(n) - Get the hi byte value of the specified replacement variable.

Example: hi(2) would get the high order byte from the second

wildcard replacement.  this will overlay the accumulator

|n    - Or the contents of the n'th replacement variable of the accumulator

WITHOUT first shifting the accumulatr

|n<v  - This is like |n, except before the value is ORed, it is shifted v

places to the left. v is a single hex digit.

|n>v  - This is like |n, except before the value is ORed, it is shifted v

places to the right. v is a single hex digit.

&nn   - The accumulator is ANDed with the hex value nn.

%     - Swap the high and low nybbles

\Bn   - Create a memory buffer.  n specifies which variable holds the size.

\En   - Set last defined label to specified variable contents

This is used for EQU instructions

\On   - Set assembly origin address to specified variable

This is used for ORG instructions

\P    - Mark the last symbol as being public.  This generates = lines.

\Q    - This can be used to mark the end of a public procedure when another

public procedure will not immediately follow.

\R    - Mark the last symbol as being a public procedere.  This generates

# lines.  All lines up to the next \R command will considered part

of the procedure

\Sn   - Specify program start address to specified variable

This is used for END instructions

\X    - Mark the last symbol as being external

Assembler directives:

---------------------

The assembler directives EQU, ORG, DB, DS, END or no long directly

recognized by the assembler.  If these directives are required, they must

be defined in the def file.  Here is the common definition for these

directives:

DB      \L

\1

DS	\W

\B1

END     \W

\S1

EQU     \W

\E1

ORG     \W

\O1

asm.rc file

-----------

The asm.rc file if it exists, will be read for options to be passed to

the assembler.  Any of the standard command line options may be placed

into this file, one option per line.  This file is read after the

initial values are set and before the command line options.

Output file

-----------

The standard output file is an ascii file representing the assembled

machine code.  The first character of each specifies what type of line

the line is.  Any programs designed to read these files should completely

ignore lines beginning with symbols they do not know, since there is the

possiblity of new line types in the future.  The current line types are

currently defined:

: - Data line

Immediately following the : is a a-4digit hexadecimal number, this is

the address in memory where the following bytes go.  Following the

address are up to 16 2-digit bytes, separated by space.  These bytes

are to be loaded starting from the specified addres.

* - Start Address

This symbol specifies where the start address of the program is.

Following the * is a 4-digit hexadecimal number representing the

starting offset for the program.

? - Missing reference (word)

This symbol is used to mark where missing references were in the source

file for which the symbols were declared external.  This can be used

by a library linker in order to locate where external references need

to be filled in.  Following the ? is the name of the external symbol.

The linker should remove this line and modify the first bytes (not

address) of the following : line.

- - Missing reference (byte)

This symbol is used to mark where missing references were in the source

file for which the symbols were declared external.  This can be used

by a library linker in order to locate where external references need

to be filled in.  Following the ? is the name of the external symbol.

The linker should remove this line and modify the first bytes (not

address) of the following : line.

= - Public symbol

This line is used to mark where public symbols are.  It is used by

the linker to provide the value for a symbol.  Immediately following the

= is the 4-digit value of the symbol followed by a space and the symbol

name. Unlike the # marker, this specifies a symbol for which no code

is copied.

# - Public procedure

This line is used to mark the beginning of a public procedure.  During

linking, the linker will include in the output file all : lines up

to the next # line.  The linker will place this procedure at an address

that is higher than all bytes written to by the source file.  The value

of the symbol will then be the address it was loaded at.  This symbol

is normally only used in library files.  Immediately following the #

is the name of the symbol.

& - Library to search

These lines are used to specify specific libraries to be searched when

looking for undefined symbols.  Immediately following the & is the

filename of the library to search.

Expression Evaluation:

----------------------

1: .0  .1

2: *   /

3: +   -

4: <<  >>

5: &

6: |   ^

Preprocessor:

-------------

#DEFINE symbol value

#UNDEF symbol

#IFDEF symbol

#IFNDEF symbol

#ELSE

#ENDIF

Embedded .def files:

--------------------

Rc/Asm now supports embedded def files.  These allow you to switch the

assembly definitions midstream  for example:

MI=VE (3DD)

V0:V2=MI

V0=1E

V3=0C

; now switch to 1802 assembler

ASMloop:	1802{

ldi 0ch

plo r4

seq

sep r4

req

}

; change back to CHIP8 and continue

I=035E

V4=18

V5=18

DO loop1

DO math2

This is code for chip-8, but midway through the assembly the 1802 .def file

is used.  When specifiying an embedded .def file, use the name of the .def

file without the .def extension immediately followed by the { charater.

There should be nothing on the line following the { character.

To close the embedded assembly you use the } character.  This character must

be on a line all by itself.